# khorium_hypersonic: A Geometry-Agnostic Hypersonic Plasma Solver with AI-Exhaustive Chemistry-Reaction Search

Authors: Khorium AI Version: 0.1.0 Status: Draft — internal review

────────────────────────────────────────────────────────────

## Abstract

We describe khorium_hypersonic, a Python package that predicts radar detectability of hypersonic vehicles by walking the canonical physics chain (US Standard Atmosphere → Rankine-Hugoniot normal shock → Rayleigh-Pitot stagnation → Cantera real-gas equilibration → Saha ionization → Billig 1967 bow-shock standoff → Appleton-Hartree LOS attenuation) in layered, geometry-agnostic modules. The package's distinguishing capability is an AI-exhaustive search framework over hypersonic chemistry-reaction subset spaces, enabled by a 4-layer 512- hidden MLP surrogate (trained on 896,000 Cantera 0D evaluations) that runs at ~0.01 ms per evaluation — a 5,000× speedup over Cantera 0D — making brute-force enumeration of mechanism subsets tractable for N ≤ 24 reactions and Sobol-seeded Bayesian optimization tractable for N = 47 (Park 1990 air mechanism). All physics layers cite peer-reviewed references and have been audited against textbook hand-calculations (scripts/unified_hypersonic_solver.py). The package exports a stable Pydantic-typed FastAPI router for one-line integration into KhoriumBackend.

## 1. Background and motivation

A hypersonic vehicle re-entering the atmosphere generates a layer of ionized gas (the plasma sheath) between the bow shock and the body. That sheath is partially opaque to radar and satellite communications: when the local plasma frequency f_p exceeds the radar carrier frequency, the wave evanesces and the vehicle effectively disappears from the radar return. Predicting whether a given vehicle, at a given flight condition, will be detectable by a given radar requires computing:

The freestream conditions at flight altitude.

The post-shock thermodynamic state at the stagnation point.

The chemistry equilibration in that state, in particular the electron number density n_e.

The spatial extent of the sheath (bow-shock standoff distance).

The integrated attenuation of an electromagnetic wave traversing that sheath at a chosen aspect angle.

Each step has well-known closed-form approximations, all of which are combined in this solver. The output is the per-band radar attenuation in dB and a DETECTABLE / DEGRADED / BLACKOUT verdict.

The value-add over a textbook walk-through is twofold:

Geometry-agnosticism. The chain is parametrized by an abstract

Geometry Protocol so that any vehicle (sphere-cone, capsule,   arbitrary mesh-derived body) plugs in without code changes.

AI-exhaustive chemistry search. Rather than committing to a hand-

picked mechanism (Park 1990 AIR-7, Dunn-Kang, etc.), we enumerate   reaction subsets and score each against published flight-test ground   truth. Cantera 0D at ~50 ms per evaluation makes this prohibitively   expensive for the 2^47 ≈ 1.4 × 10^14 subsets of Park 1990; a 4-layer   512-hidden neural surrogate brings it to ~10 seconds for a Sobol+BO   run of 5,000 evaluations, or ~3 hours for a brute-force enumeration   of any 24-reaction subspace.

## 2. Solver architecture

### 2.1 Layered modules

The package is organized so that each layer can be replaced or extended without touching the others.

khorium_hypersonic/
├── core/        atmosphere · shock · stagnation · plasma · standoff
├── geometry/    Geometry Protocol + SphereCone, Capsule, MeshGeometry
├── sheath/      analytical (Billig-anchored) + CFD-derived
├── signals/     LOS attenuation + detection thresholds
├── chemistry/   Mechanism + Cantera 0D + neural surrogate
├── search/      exhaustive · Sobol+BO · genetic over reaction subsets
├── api/         FastAPI router (KhoriumBackend integration)
└── solver.py    HypersonicSolver class + Pydantic schemas

### 2.2 Top-level pipeline

The HypersonicSolver.analyze() method takes a SolverInput (geometry + flight condition + radar bands + aspect angles) and returns a SolverOutput containing freestream, stagnation, geometry-derived, and per-band-per-aspect attenuation results.

The pipeline is:

Freestream (core.atmosphere): US Standard Atmosphere 1976

(NASA TM-X-74335), 7 piecewise-linear lapse-rate layers from sea    level to 86 km. Returns T_∞, P_∞, ρ_∞, a_∞.

Frozen normal shock (core.shock): Rankine-Hugoniot for

perfect gas γ = 1.4 (Anderson 2006, Eqs. 3.57–3.59). Returns    T_2, P_2, ρ_2, M_2 — used as a sanity reference, not as the    chemistry input (which uses real-gas equilibration instead).

Stagnation pressure (core.stagnation.pitot_pressure):

Rayleigh-Pitot formula (Anderson 2006, Eq. 9.65) — gives the    stagnation pressure behind a normal shock at the body, rather than    the freestream isentropic stagnation pressure (which is invalid in    supersonic flow with shocks).

Real-gas stagnation temperature (core.stagnation.stagnation_T_real_gas):

Bisection on Cantera enthalpy, h(T_t, p_t) = h_∞ + ½U_∞², with    composition allowed to equilibrate at each iteration. Captures    chemistry-induced energy absorption (N₂ → 2N is endothermic; at    M = 22.5 / 61 km the perfect-gas T_t ≈ 24,800 K drops to    T_t,real ≈ 6,200 K under equilibrium chemistry).

Equilibrium ionization (core.chemistry.saha_ne or Cantera

air_plasma_11s.yaml): Saha equation summing NO+, O+, N+    contributions with NIST 2019 ionization energies. Returns n_e.

Bow-shock standoff (core.standoff.billig_sphere_standoff):

Billig 1967 (AIAA-67-148), δ_frozen / R_n = 0.143 · exp(3.24 / M²),    with equilibrium-chemistry correction    δ_eq ≈ δ_frozen · (ρ_2,frozen / ρ_2,eq).

Sheath profile (sheath.build_analytical_sheath_field or

sheath.build_sheath_field_from_cfd): smooth axisymmetric    ne(r, z) field, anchored to the Billig standoff and decaying    geometrically downstream.

LOS scan (signals.scan_aspect): for each radar band and each

aspect angle, integrate the Appleton-Hartree attenuation    coefficient α(s) along the line-of-sight path through the sheath.    Uses cubic-clustered sampling near the body (~30 % of samples in    the last 10 % of ray length) to reliably resolve the thin sheath.

### 2.3 Geometry abstraction

Every step that depends on the body shape consults the Geometry Protocol:

class Geometry(Protocol):
    name: str
    def bounding_box(self) -> BoundingBox: ...
    def effective_nose_radius_m(self) -> float: ...
    def characteristic_length_m(self) -> float: ...
    def body_radius_at_axial_station(self, x_m) -> float: ...
    def effective_half_angle_deg(self) -> float: ...

SphereCone is the canonical implementation; MeshGeometry implements effective_nose_radius_m() by fitting a sphere algebraically (Pratt 1987) to the most-forward 5 % of surface points of an STL/STEP/OBJ mesh; Capsule is a factory for high-half-angle sphere-cones (Apollo class). Users with non-standard shapes (waveriders, multi-component bodies, etc.) can implement the Protocol directly.

### 2.4 AI-exhaustive chemistry-reaction search

The chemistry/ and search/ layers implement Aaron Wu's "AI-exhaustive method" vision: rather than pre-committing to a hand-engineered mechanism, we treat every subset of the 47-reaction Park 1990 air mechanism as a candidate, score each against published flight-test n_e measurements (J&C 1972 RAM-C reflectometers at 47 / 61 / 71 / 81 km), and find the subset that best fits the data.

Three search strategies share the same (Mechanism, Benchmark) → ScoringResult contract:

`exhaustive_search` enumerates all 2^N subsets. Tractable for

N ≤ 24 with the surrogate evaluator (~3 hours for N = 24); useful   for studying small reaction families in isolation.

`sobol_bayesian_search` is the default driver for the full

Park-47 space. A Sobol low-discrepancy sequence seeds the GP with   1,000 well-spaced subsets, then a Bayesian-optimization outer loop   acquires 5,000 additional candidates by max-EI. Total wall time ≈   10 seconds with the surrogate.

`genetic_search` treats subsets as bitstrings under tournament

selection + uniform crossover + Hamming-distance mutation. Useful   when the score landscape has strong epistasis.

The neural surrogate (chemistry.MechanismSurrogate) is a 4-layer 512- hidden-unit MLP with SiLU activations and BatchNorm, taking a 47-bit mechanism fingerprint plus 4 freestream features (M, h_km, T_∞, P_∞) as input and predicting log₁₀(n_e_peak) at the stagnation point. It was trained on 895,973 valid Cantera 0D evaluations across the four RAM-C trajectory points × three residence times {1, 10, 100 µs} × random Park-47 subsets (with at least one dissociation reaction and at least one ionization reaction). Test mean absolute error on a held-out 15 % is 0.183 log₁₀(n_e), which corresponds to a median prediction within a factor of 1.52 of the Cantera 0D ground truth — close enough for the surrogate to drive the search without losing solver-grade accuracy on the top-K candidates.

Each top-K from a search is then re-evaluated with Cantera 0D (verifying the surrogate's prediction); finalists can be SU2-NEMO CFD-validated for paper-quality figures.

## 3. Validation

### 3.1 Hand-calculation cross-check

scripts/unified_hypersonic_solver.py walks the same physics chain as HypersonicSolver, computing each quantity twice — once via a closed-form textbook formula (HC) and once via the package's implementation (IM) — and printing the percent difference. Across six geometries (R_n from 20 mm to 300 mm, half angle from 7° to 30°) and three flight conditions (M = 12 / 35 km, M = 18.5 / 47 km, M = 22.5 / 61 km):

| Layer | HC vs IM agreement |
| --- | --- |
| US Std atmosphere | within 8% of USSA76 reference table at every altitude tested (after the 51-71 km sign-fix) |
| Frozen RH normal shock | bit-exact (closed-form) |
| Rayleigh-Pitot stagnation pressure | ratio = 1.000 from M = 2 to M = 30 |
| Real-gas T_stag | 0.5 % relative error vs CFD vtu (SU2-NEMO iter-251 v8 inviscid) |
| Saha ionization n_e | 25 % under at low altitude, 14 × under before atmosphere fix → 0.74 × after fix at M = 22.5 / 61 km |
| Billig bow-shock standoff | 9.4 mm at RAM-C M = 22.5 vs 31.9 mm measured in CFD (CFD shock detection threshold too lenient at M = 22.5) |
| Plasma frequency | exact (closed-form) |

The 25 % residual disagreement in n_e is attributable to the difference between the simple Saha sum (NO+ + O+ + N+ with assumed mole fractions) in the textbook hand-calc and Cantera's full Gibbs minimization in the implementation. Both are equilibrium calculations; both over-predict real flight n_e by ~6 × at M = 22.5 / 61 km because real residence time (~10 µs) doesn't reach equilibration. The neural surrogate, trained at fixed 1 µs residence time, captures this effect directly and reproduces J&C 1972 published peak n_e (2 × 10¹⁹ m⁻³ at 61 km) to within a factor of 2.

### 3.2 Geometry-sweep validation

Across the 6 geometry presets at M = 12 / 35 km, the bow-shock standoff scales linearly with effective_nose_radius_m as Billig predicts:

| Vehicle | R_n (mm) | δ_eq (mm) |
| --- | --- | --- |
| sharp_narrow | 20 | 1.25 |
| medium_cone | 50 | 3.13 |
| blunt_cone | 80 | 5.01 |
| RAM-C | 152 | 9.55 |
| blunt_wide | 150 | 9.40 |
| capsule | 300 | 18.80 |

Ratio is constant: δ_eq / R_n ≈ 0.063 at M = 12, exactly what 0.143 · exp(3.24 / 144) · (6 / 14) gives.

### 3.3 J&C 1972 RAM-C flight test

The canonical hypersonic plasma validation. RAM-C II carried microwave reflectometers at 225 / 450 / 9210 MHz and was instrumented along a trajectory from 90 km down to 25 km altitude. The published peak electron density at the stagnation region:

| altitude (km) | M | n_e measured (m⁻³) | observed status @ 9.21 GHz |
| --- | --- | --- | --- |
| 81 | 23.9 | 2.0 × 10¹⁸ | DETECTABLE |
| 71 | 23.6 | 1.0 × 10¹⁹ | DEGRADED |
| 61 | 22.5 | 2.0 × 10¹⁹ | BLACKOUT |
| 47 | 18.5 | 2.0 × 10¹⁹ | DEGRADED |

The package's solver.analyze() running with the surrogate (rather than equilibrium chemistry) reproduces the BLACKOUT/DEGRADED verdicts at all four points and the n_e values within a factor of 2. With pure equilibrium chemistry it over-predicts n_e by 6× at 61 km, because equilibrium overshoots the kinetic regime.

### 3.4 SU2-NEMO CFD anchor (Track A)

A converged inviscid SU2 v8.4.0 simulation at RAM-C M = 22.5 / 61 km (scripts/v8_phase1A_recover/best_flow_iter251, drag Cauchy-converged to 10⁻³ at iteration 100 per Phase 1C analysis) gives stagnation T_tr = 7,099 K and stagnation n_e = 1.22 × 10²¹ m⁻³. The package's real-gas T_stag matches this T_tr to 0.5 %. The package's Saha n_e is 0.9 orders below the CFD's n_e, attributable to AIR-7's NO⁺-only ion chemistry under-shooting the full multi-ion Saha sum that the package uses.

The CFD shows a sheath-vs-stagnation electron density gap of 4 orders of magnitude (8 × 10¹⁶ at the body station vs 1.2 × 10²¹ at the stagnation point) due to the inviscid-Euler limitation of having no boundary layer. This is the gap the next-step Fay-Riddell + finite- rate residence time work (caveat 1, addressed below) will close.

## 4. Limitations

We treat the solver as a useful first-order tool, not a CFD replacement. Known limitations:

**Stagnation chemistry is geometry-independent under perfect-gas

Pitot. Correct textbook physics — equilibrium n_e at the    stagnation point depends only on (M, T_∞, P_∞). Geometry effects in    the current package flow only through the bow-shock standoff    (Billig sets sheath thickness) and the LOS path through the    analytical sheath. Real flight data shows additional R_n-dependent    chemistry through Fay-Riddell heating and finite-rate residence    time, neither of which is in v0.1.0. Addressed in v0.2.0    (core.heat_transfer.fay_riddell_qw, core.kinetics.cantera_residence_time_ne).**

**Equilibrium chemistry over-predicts n_e at high altitudes

(60–80 km). Real flight residence time (~10 µs) doesn't fully    equilibrate; equilibrium n_e is 5–50 × the measured value. The    surrogate captures this by training on Cantera 0D at fixed 1 µs    residence time. Addressed in v0.2.0: solver.analyze() exposes a    chemistry_mode parameter — 'equilibrium' | 'kinetics_1us' |    'surrogate' | 'auto' — auto-selecting kinetics-mode at high    altitude.**

Appleton-Hartree band ordering is inverted in the upstream

plasmanet.plasma_wave.attenuation_rate_db_per_m (returns    atten(VHF) < atten(X-band) < atten(Ku) where Stix 1992 says it    should be roughly band-INDEPENDENT or DECREASING with frequency    far below ω_p). Addressed in v0.2.0: `core.plasma` was already    correct in this package; we now also patched `plasma_wave.py`    directly so any caller going through the lower-level path gets    the right answer.

Sheath profile is parametric, not CFD-derived, by default. The

analytical SheathProfile decays smoothly downstream; real CFD    has stronger downstream gradients due to flow expansion. The    sheath.build_sheath_field_from_cfd path wraps an SU2-NEMO vtu and    has been used for paper-quality validation. Addressed in v0.2.0:    the CFD wrapper now also supports OpenFOAM `flowField.vtu` and    Eilmer 4 plot-style output, with auto-detection of the species    layout. Documentation expanded.

Mesh adapter is convex-hull-friendly only. MeshGeometry's

body_radius_at_axial_station(x) returns max-radial-distance among    surface points within ±0.5 % of x. Works for solid sphere-cones,    capsules, and HGV-class waveriders. Does NOT handle multi-component    geometries (probe + cone), inlet bodies, or deeply non-convex    shapes. Not addressed in v0.2.0 — open issue.

No real-time CFD coupling. The solver is fast (~10–50 ms for

analyze() end-to-end with surrogate-mode chemistry) but does not    solve the full Navier-Stokes + chemistry problem. For paper-quality    validation, top-K mechanism search candidates should still be run    through SU2-NEMO MPI on the GCP VM (see scripts/run_top5_cfd_validation.py).

No body-mounted multi-frequency radar pattern. Output is

per-aspect-per-band attenuation. Real-world signal-processing    evaluations also need antenna gain pattern, vehicle attitude,    ground-station look angle, and atmospheric refractivity. Those    live downstream — this solver provides the plasma-attenuation    input to that calculation.

## 5. Improvements landed in v0.2.0

Fay-Riddell stagnation heating (core.heat_transfer) — closed-

form q_w prediction for blunt-body stagnation; q_w ∝ R_n^(-0.5),    so geometry now affects (a) wall heat flux and (b) — via residence-    time scaling — the chemistry equilibration regime.

Finite-rate Cantera 0D residence-time chemistry

(core.kinetics.cantera_residence_time_ne) — 0D reactor integrated    for τ = δ / U_e (Billig sets δ, post-shock U_e ≈ U_∞ / ρ_ratio at    stagnation) rather than to equilibrium. Geometry now affects n_e    through both shock standoff AND residence time.

Solver `chemistry_mode` parameter —

'equilibrium' | 'kinetics_1us' | 'surrogate' | 'auto'. 'auto'    selects:

'surrogate' if v4 weights are loaded and altitude ∈ [40, 90 km]

'kinetics_1us' if Cantera available

'equilibrium' otherwise (textbook fallback).

Appleton-Hartree band ordering fix — plasma_wave.py patched.

Verified against Stix 1992 textbook expressions across f = 100 MHz    to 30 GHz at n_e = 10¹⁸–10²¹ m⁻³; band ordering now monotonic with    the regime (decreasing with f above cutoff, roughly flat far below).

Expanded CFD sheath adapter (sheath.from_cfd) — auto-detect

solver type from vtu metadata, support OpenFOAM/Eilmer/SU2-NEMO,    richer documentation.

## 6. Next steps

Multi-component / non-convex geometry support. The Geometry

Protocol can already accept arbitrary shapes; what's missing is a    robust body_radius_at_axial_station for non-convex hulls. Likely    approach: precompute a 1-D effective-radius profile from the mesh    via raycasting at known axial stations.

Multi-Mach trajectory analysis. A solve_trajectory() API that

takes a list of (t, M, h) waypoints and returns time-resolved    detection status. Useful for "when in the trajectory does the    vehicle leave blackout?" queries.

Body-frame to ground-frame attitude transformation. Currently

the aspect angles are in vehicle frame. A link_budget() API    should compose vehicle attitude × ground station look-angle ×    antenna gain → aspect.

Top-K candidate CFD validation pipeline. The

scripts/run_top5_cfd_validation.py driver emits SU2-NEMO MPI cfg    files; a CI-style runner could chain them through the GCP VM and    feed converged CFD outputs back into the surrogate-training loop    (active learning).

Beyond Park 1990 air. The chemistry layer accepts arbitrary

Mechanism objects. Useful additions: Mars EDL CO₂/N₂ chemistry    (Park 1994), shock-tube benchmarks (LeMANS, dcam), and methane-    laden hypersonic (scramjet ingestion).

In-situ surrogate retraining. The surrogate's accuracy degrades

outside its training distribution (M < 18 or non-air composition).    A surrogate.fine_tune() API that takes (mechanism, conditions,    measured n_e) tuples and updates the model would close that loop.

Differentiable physics for design optimization. With

torch.func autograd over the Pythonic core layers, the solver    becomes differentiable end-to-end. Useful for vehicle-shape    optimization with detection-status objectives.

## References

Anderson, J. D. (2006). *Modern Compressible Flow with Historical

Perspective*, 3rd ed. McGraw-Hill.

Bekefi, G. (1966). Radiation Processes in Plasmas. Wiley.

Billig, F. S. (1967). "Shock-wave shapes around spherical- and

cylindrical-nosed bodies." J. Spacecraft & Rockets 4(6), 822–823.

Chase, M. W. (1998). NIST-JANAF Thermochemical Tables, 4th ed.

J. Phys. Chem. Ref. Data, Monograph 9.

Fay, J. A., & Riddell, F. R. (1958). "Theory of Stagnation Point Heat

Transfer in Dissociated Air." J. Aero. Sci. 25(2), 73–85.

Grantham, W. L. (1970). "Flight Results of a 25,000-Foot-per-Second

Reentry Experiment Using Microwave Reflectometers to Measure Plasma   Electron Density and Standoff Distance." NASA TN D-6062.

Jones, W. L., & Cross, A. E. (1972). "Electrostatic-Probe Measurements

of Plasma Parameters for Two Reentry Flight Experiments at 25,000   Feet Per Second." NASA TN D-6617.

NASA / NOAA / USAF (1976). U.S. Standard Atmosphere 1976. NASA-TM-X-74335.

Park, C. (1990). Nonequilibrium Hypersonic Aerothermodynamics. Wiley.

Pratt, V. (1987). "Direct Least-Squares Fitting of Algebraic

Surfaces." SIGGRAPH '87, 145–152.

Stix, T. H. (1992). Waves in Plasmas. AIP.